ADVANCES MADE IN BRITISH COMPUTING TECHNOLOGY DURING AND IMMEDIATELY AFTER THE SECOND WORLD WAR

Robert McMordie

This document is copyright 1997-98 Robert McMordie.

The text in this document may not be reproduced in any form without my prior consent. I can be contacted at r.j.mcmordie@bcs.org.uk.

Document revision 1 (5th November 1998).

ADVANCES MADE IN BRITISH COMPUTING TECHNOLOGY DURING AND IMMEDIATELY AFTER THE SECOND WORLD WAR

1. INTRODUCTION

The Second World War was a time of great innovation in the field of computing. Before the start of the war, the computer as we know it today did not exist. The computers of the time were almost all mechanical calculating machines, designed for arithmetic operations. By the end of the war, however, the first general purpose computer had been operational for over a year and a half, and the foundations of modern computing had been laid. Nor did the advances in technology stop then. In the immediate post-war years, numerous advances were made in this field.

In this essay, I will look at the advances made in computing during the war years, starting with the bombes and the Robinson machines, and moving to Colossus. I will also look at the technology available in the immediate post-war years and then examining briefly the three main post-war developments, the NPL Pilot ACE, the EDSAC and the Manchester Mark I.

2. THE SECOND WORLD WAR

The Second World War was a period of frantic development in the field of computing. In Britain, this development was mainly focused on cryptanalysis, in an attempt to break the German 'Enigma' and 'Tunny' (known to the British as 'Fish') codes 9. The development was mainly based at the Government Code and Cypher School at Bletchley Park and the General Post Office Dollis Hill Research Division, in London. Before looking at the developments made in cracking these codes, it is useful to know some background about the codes, and how the cryptanalyists set about cracking them.

2.1 ENIGMA

The Enigma machine was originally developed in 1919 and was adopted by the German Armed Forces between 1926 and 1935 9, following a number of improvements. It was actually based on an earlier design by an American, named Heburn 11. The machine consisted of a box, which when opened revealed a standard, German 26 key typewriter style keyboard. Above this, was an array of 26 bulbs, representing the possible outputs. Below the keyboard was a plug board. Figure 1 shows an Army Enigma machine.

Setting up an Enigma machine so that it was ready to encode or decode a message was quite complex. Three 'wheels' had to be selected from a choice of five (or eight, in the Naval version), and arranged in a specified order in the machine. The starting position of each wheel then had to be selected from the available positions. Finally, the plug board had to be set up. The machine was then ready to be used.

The Enigma machine constructed an encrypted message using substitution techniques 9. To encrypt a message, the operator had to choose a three letter 'indicator setting', at random, by setting the starting positions of the three wheels. A three letter 'message setting' was then chosen, again at random, and keyed in twice. This resulted in a six letter 'indicator'. The wheels were then set to the message setting, and the text to be encrypted was entered. This information and the message was then transmitted.

To decrypt a message, the decoding machine was set up in the same way as the encrypting one. The wheels would be moved to the indicator setting, and the indicator entered. This would result in the message setting being returned. Due to the nature of the Enigma machine, if pressing A gives a G, pressing G will give A. The wheels are then set to the message setting, and the encrypted message entered. The clear text message is then returned.

The reason the Enigma codes were so difficult to break was that there were approximately 9 x 1020 initial settings for a standard Enigma machine 11. It was completely infeasible to run through every single setting by hand, just to try and read one message. It was for this reason that the bombe was developed.

2.2 THE BOMBE

The Bombe was an electro-mechanical relay device that could run through all the possible initial settings of an Enigma machine. There are at least two different stories of how this machine was named. The first story is that the two inventors were eating an ice-cream bombe when the idea was first discussed 14. The second is that when the machines were operating, they made a ticking sound, like a bomb 15. The bombe was initially developed in Poland. The Polish had begun trying to decypher Enigma codes during the early 1930's, and were helped by Hans-Thilo Schmidt, who sold a large amount of information about Enigma to the French between 1931 and mid-1939. In July 1939, the Polish called a meeting with the British and French intelligence agencies, and revealed that they had managed to break the Enigma code. The British were given a copy of the Enigma machines and descriptions of the bombes. A number of British bombes were then built at Bletchley Park, featuring a number of innovations made by Gordon Welchman and Alan Turing. Details of these have not been disclosed, but they apparently resulted in a large increase in the power of the bombe.

Each bombe was approximately 8 feet in height, and seven feet wide 9. On the front were a number of drums, corresponding to the wheels in an Enigma machine. Inside the drums were a large number of wire brushes. The rear of the machine was a mass of wiring. The drums and the wiring had to be set up according to a 'menu'. The wire brushes inside the drums had to be adjusted with tweezers, to avoid electrical short circuits. The bombes were difficult to set up, generated a lot of heat and noise and were slow. There was also the danger of severe electric shocks. However, they played a very important role in the decyphering of Enigma messages, and paved the way for later computing developments.

3. THE FISH CODE

3.1 INTRODUCTION

The Fish code was developed to encrypt non-Morse Code messages, to be sent by radio 9. The messages were entered using a keyboard or paper tape, and were represented using the International Teleprinter Code, or Baudot Code. This is a code that can be used to represent the 26 letters of the alphabet together with six 'control' symbols, such as space, line feed, etc. using an array of five units, or bits. The message was then simultaneously encrypted and transmitted. At the receiving end, the message was simultaneously received and decrypted.

There were several different versions of Fish. The version referred to by 'Fish' in this essay is the Schlüsselzusatz ('cypher attachment') version 8, named 'Tunny' by Bletchley Park. The other was called Geheimschreiber ('secret writer').

The Germans started using the Fish code on a regular basis in mid-1941 9. The Fish machine was based on a different encryption technique to Enigma and the transmissions were high speed. This meant that very accurate timing information was needed, to be able to correctly decypher messages. However, being able to read these messages was of major importance. Enigma was mainly used for operational and tactical communications, mainly within units and occasionally between units, for example within one army or between two armies. For any communications above unit level, such as to Commanders-in-Chief, Fish was used. Fish messages usually contained situation reports, orders, and information about the general state of the German armies.

The encryption method was based on modulo-2 addition of characters 9. The basic system was first proposed in 1918 by American Gilbert S. Vernam 8. He suggested a machine that would take a stream of plain text from a paper tape, and a stream of random letters from a 'key' tape. For each pair of characters read in, the bits would be added together using modulo-2 addition. The resulting character would then be transmitted. The proposed method would have the advantage that it would be self-reversing, so only one type of machine would be needed for encryption and decryption operations. For example, consider the encryption and decryption of the letter 'N', if the key letter was 'D'. The Baudot codes for N and D, represented in standard binary notation, are 00110 and 10010.

Encryption :		0	0	1	1	0	= N

1	0	0	1	0	= D (key)

1 	0	1	0	0	= S (encrypted letter)

Decryption :		1	0	1	0	0	= S (encrypted letter)

1	0	0	1	0	= D (key)

0	0	1	1	0	= N (decyphered letter)

The system was not without its problems, however. The encoder and decrypter of a message needed to have the same tape in the machine to be successful. Each tape could also only be used once, and had to be created with care, to avoid generating patterns in the letters.

The system employed by the Germans was similar to Vernam's suggested system, but instead of using a key tape had a system comprising twelve wheels, which generated random characters. These wheels were divided into five pairs, two for each character, and two 'driving' wheels. A schematic diagram of this arrangement is given as figure 2 at the end of the essay.

The first wheel in each pair was called the K wheel and the second the S wheel. The numbers written inside each wheel on the diagram represents the 'length' of each wheel, the number of times each wheel could be turned before it made a complete revolution. The K wheels moved every time a letter was entered, and the starting position could be chosen by the operator. The movement of the S wheels was much more complex, as they were affected by the movements of the motor wheels. When they did move, all five S wheels moved together. The design of the wheels was also different to the Enigma wheels. The wiring was not fixed, but was determined by the positioning of a number of two-position pins, located around the twelve wheels. The number of pins on each wheel was equal to the wheel length. As a result of this, there were 501 pins that needed to be individually set 9. If a pin was set in one position, the bit input would be reversed (using logical NOT) and if it was set in the other, the input would not be reversed.

3.2 TUNNY

The methods needed to break the Fish code were much more complex than the Enigma code. The initial break for the cryptanalyists trying to decypher the Fish code came when a German cypher clerk encrypted a long message twice, using the same initial wheel settings 9. The operator had typed the message in on the keyboard, but it had not been correctly received. The operator then had to type the message again. As a result, there were differences between the two copies of the message - the second message was roughly 500 characters shorter, due to extra abbreviations made by the cypher clerk.

The cryptanalysts at Bletchley Park were able to eliminate the random characters that had been added by the Fish machine, and were left with a stream of text that was actually the two plain text messages added together using modulo-2 addition. This was given to Brigadier John Tiltman 9, who was able to split the text stream into the two original messages. Finally, one of the original messages was added to both encrypted messages. Two streams of characters were extracted, the added characters and a purely random stream of characters. The streams of characters were then passed on to W. T. Tutte, who was able, by logical analysis, to deduce a number of pieces of information about the Fish machine, including the machine's structure and the lengths of all the wheels.

From this information, the cryptanalysts at Bletchley Park were able to build their own version of the Fish machine, that would allow them to decypher the German messages. This was called Tunny 9. While logically it was very similar, physically the British machine was very different. The Fish machine was mainly mechanical, but the Tunny was electrically based, with the German wheels being emulated using electrical switches and pairs of cables, one for input and one for output. Before it could be used the British machine had to be set up, by plugging the wires into an array of sockets, representing the teeth of the wheels.

3.3 DECYPHERING THE CODE

Decyphering the code was the final step in a chain of operations. Before decyphering was possible, the starting positions and wheel settings (or patterns) of the wheels of the encrypting machine had to be found. These two processes were called wheel setting and wheel breaking 9.

The details of the mathematics behind these two processes are beyond the scope of this essay. It relied partly on the fact that previously (hand) decoded messages contained a proportion of repeated characters that was greater than would be expected by random chance 8. Modulo-2 arithmetic was used, along with a range of statistical techniques.

The basic method for finding the patterns required a long sequence of code 8. First, the entire code was displaced right one position. The bit on the end wrapped round. An addition table, usually represented as an array of cells, was then constructed. The dimensions of the grid were determined by the number of teeth on the pair of wheels currently being broken. In this example for wheels K1 and K2, the grid would be 41 by 31. Each of the cells represented one of the wheel positions. The code was looked through bit by bit, and the grid was filled in following a particular order. This order was based on the number of positions of each wheel. Each cell could have more than one entry in it. Each individual entry was determined by modulo-2 addition of the bit from the original sequence and the bit from the displaced sequence. The resulting bit was entered into the cell.

When the entire code had been processed in this manner, each cell was analysed to see which type of bit was in the majority. The number of this type of bit was then summed, to give a magnitude. So, if a cell contained 12 entries, consisting of 8 '0's and 4 '1's, '0's would be in the majority and the magnitude would be 8. If it contained 8 '1's and 4 '0's, '1's would be in the majority and the magnitude would then be -8. If the number of each type of bit was equal, the magnitude was 0. These values were entered into a new grid, of the same size. This new grid then shows the majority counts of '0's over '1's.

Two sets, DK1 containing 41 distinct values, and DK2 containing 31 distinct values, were then considered, and an addition table was then constructed. DK1 contained information about the same row as a particular cell, and DK2 information about the same column as a particular cell. The table contained the same information as the previous table, but represented different information, approximations for the new addition table entries.

It was then possible to estimate the values of DK1 using DK2 and the table. This was done by first assuming a set of values for DK2. This was usually achieved by looking at the row of the table containing the largest majority values, and putting a '0' in DK2 if the value was positive and a '1' if negative. The following algorithm was then applied 8 :

1) Consider each column in the table. If the corresponding DK2 symbol is a '1', change the signs of all the numbers in that column.

2) Sum all the numbers in the corresponding row.

3) If the sum is > 0, the result is a '0', otherwise it is a '1'.

The resulting estimate of DK1 can then be used to refine the estimate of DK2, using the same method. The repeated alteration of DK1 and DK2 can then be used to get better and better estimates. This then resulted in the 'correct' wheel patterns for a pair of wheels, and had to be repeated for all five wheels.

The method used to determine the start positions of the K wheels involved probability analysis of the encrypted code and is beyond the scope of this essay. It involved finding the sets of wheel patterns and then applying these to 'pseudo plain text', obtained from the encrypted code. A score was then allocated to the result, by counting the number of '0's generated by a formula that depended on the initial K wheel settings 8. The higher the score, the more accurate the settings were likely to be. This information was then given to another section at Bletchley Park, for actual decryption.

3.4 THE ROBINSON MACHINES

Wheel setting and wheel breaking took a long time to complete by hand, and one of the members of the Testery at Bletchley Park, a section set up to exploit the breakthrough made in the code, M. H. A. Newman, proposed that the non-linguistic processing could be automated and performed using electronic machinery. The result was the Robinson machines 9.

The first Robinson machine was called Heath Robinson, after the British artist famous for his designs of strange machines. It was a combination of relays and valves, with input coming from two paper tapes which were read using photoelectric cells. Much of the technology was initially developed at the General Post Office Dollis Hill Research Division. The original purpose was to find the settings of the K wheels, after the patterns had been found by hand. The Heath Robinson design had a number of weaknesses, including problems caused by the speed of the paper tape system, which would eventually cause the tape to tear, and there were numerous problems caused by the haste with which it had been assembled, but that did not stop three other machines, Peter Robinson, Robinson & Cleaver (named after two shops in London 8) and Super Robinson 1 being constructed. The Robinson machines also helped to prove that it was possible to construct high speed machines to help with the decoding process, and although they cannot be considered to be 'true' computers, because of their highly specialised nature, they paved the way for many future developments, including the Colossus machines.

4. COLOSSUS

Alan Turing was perhaps the main contributor to the theory behind the early British computers, mainly Colossus. He was born in London in 1912 and studied mathematics at Cambridge University, passing with a distinction in 1934 1. After getting his degree, he stayed at Cambridge, hoping to eventually follow a career in research. In 1936 he had published a paper  'On Computable Numbers with an Application to the Entscheidungsproblem', proving that some problems could not be solved within a finite number of definite steps. In this, Turing described a theoretical machine, now known as the Turing Machine, consisting of a tape and a scanner. The tape would consist of a number of 'boxes' of information. The machine would be able to scan a box and stop, erase a symbol from a box and write a new one or scan a box and move either left or right. The proposed machine would have been able to solve many, but not all, logical problems. The reason why it would not have been able to solve all problems is beyond the scope of this essay. It is clear now that the Turing Machine is actually an unlimited stored-program computer 2. After two short spells in Princeton, America, between 1936 and 1938, at the outbreak of war he was recruited by the Foreign Office to work at the Government Code & Cypher School at Bletchley Park, where as well as working on the underlying theory of Colossus, he made the improvement to the bombes referred to earlier, and was the head of a section that used the bombes  to decypher Enigma messages 9.

In early 1943, Tom Flowers was called to see Newman at Bletchley Park, at the suggestion of Turing 9. Flowers was asked to attempt to redesign bits of the Robinson machines, in an attempt to improve reliability. It was quickly realised that the best way to approach this was to start again, this time with an all-electronic machine. This machine eventually became Colossus. The first Colossus machine was assembled at Dollis Hill in December 1943 and was transferred to Bletchley Park in the same month 8. By the start of 1944, it was in regular use. The purpose of Colossus was to further automate the breaking of the Fish codes. It could perform the wheel breaking operations of the Robinson machines, and initially was only used for that purpose. However, it was later realised that Colossus could also assign the scores for the results of the initial position finding operations and it was quickly utilised to do that. It was, of course, much faster than the Robinson machines could be, due to its purely electronic nature.

Colossus was located in Hut 11 at Bletchley Park 8. A separate building was needed to house the generators needed to supply the power. Approximately 1,500 valves were used, of exactly the same type that could have been found in standard General Post Office switchboards and machines of the time. The machine used Boolean logic. Input was taken from a single paper tape. The second tape that the Robinson machines required, containing information about the wheel patterns and starting positions, was not required as the information was represented internally. The tape reader was a much improved version of the one used by the Robinson machines, running about 2.5 times faster, allowing Colossus to read in 5000 characters per second. The improvements allowed the tape to be driven by pulleys, instead of a combination of pulleys and sprockets. This meant that the tape would not be stretched, and the sprocket holes could then be used reliably for synchronisation purposes, this having been a major problem with the Robinson machines. Output was by a standard typewriter arrangement.

Before being used, Colossus had to be set up, or programmed. This had to be done in three different places around the machine, using two different methods. There were two banks of plug boards, one on the back of the machine and one on the front 12. The wiring of the boards at the back was changed infrequently, but the boards at the front were altered for each different run. There were also a number of toggle switches that needed to be correctly set each time. This allowed the machine to be programmed to control the way in which it adjusted the trial cypher keys stored internally, partially by allowing some of the circuitry to be connected in parallel or in series 11

The original Colossus, later known as the Mark I Colossus, proved to be a great success, and an order for a number of Mark II Colossi was sent to Dollis Hill in March of 1944. Mainly due to the foresight of Flowers, who had anticipated the order and started manufacturing the more time-consuming components, the first of the Mark II machines was installed and operational by June 1st, 1944. The Mark II machine featured a number of improvements over the Mark I. It could read the tape at an (effective) rate of 25,000 characters per second, and was faster. It also contained 2,500 valves. Eventually, there were about ten Colossi machines in various locations 11.

Another change between the Mark I and Mark II machines was the introduction of parallel circuitry in the Mark II 11. In fact, some of the circuitry was quintupled. This was one of the reasons for the large speed increase between Mark I and Mark II machines.

Colossus was far more general-purpose than the Robinson machines. It could perform any mathematical operation that could be represented using Boolean logic, including all the standard arithmetic operations. In fact, after the war it was shown that a Colossus machine could be programmed to perform base 10 multiplication. The plug board wiring was so complex that it would not have been feasible to do this on a regular basis, and because of the amount of circuitry the calculation could not be completed between clock pulses 9. However, it did show that Colossus was more general purpose than many suspected.

Very little information about the construction of Colossus has survived. By the early 1960's, all the original plans had been destroyed. The original machine was broken up at the end of the war, by Churchill's orders, into pieces 'no bigger than a man's hand', and buried somewhere on the Bletchley Park site. However, there is currently a Colossus Rebuild Project, directed by Tony Sale, which is located at Bletchley Park. The aim is to reproduce a fully working Colossus. As of March 1997, the machine was able to take input from a paper tape, and determine the correct wheel patterns and initial positions, based on two of the five bits for each character.

Despite the fact that it was built for a specialised purpose, Colossus was a general purpose machine and can be regarded as one of the earliest, if not the earliest, digital computer.

5. AFTER THE WAR

When the war ended, many of the people who had been involved in Bletchley Park ended up in universities and it was mainly, but not exclusively, here that the next phase of development in British computing took place. Before looking at the development made, I will first briefly examine the technology available.

5.1 THE STATE OF TECHNOLOGY

The main component of all early British computers was the thermionic valve. There are a number of different types, the main ones being the diode and triode values, so named because of the number of electrodes each had 10.

A valve consists of an evacuated glass tube, containing a number of metal electrodes. These electrodes are used to control a flow of electrons, generated by heating a cathode electrode. This method gave rise to a number of disadvantages, not least of which was that the valves generated a large amount of heat due to the cathode heaters. In addition, the heaters were inefficient and used a large amount of power, and deteriorated with time, especially if the valves were turned on and off frequently. However, it was the reliability of valves that presented the greatest problems. This was a problem that Flowers needed to address with Colossus. He discovered that the valves were far more reliable if they were left on all the time, so that was how Colossus was run 9. Although transistor based computers were available from about 1956, valve computers were still common until about the middle of the 1960's.

There were a number of ways of implementing data storage in the early computers. For temporary storage within the processor unit, a number of flip-flop circuits were used, but this was expensive. A fairly common method was to use a delay-line store. In a delay-line store, information was represented as a stream of pulses, continually travelling around a closed path. The length of time for an impulse to make one circuit was chosen so that it was much longer than the time taken by electrical impulses to travel around the wires in the computer. One type of delay-line store, the acoustic delay-line store, typically consisted of a 5 ft long steel tube of mercury, covered at both ends with quartz crystal, to transmit and receive the impulses 10. The electronic pulse from the computer was transformed into an ultrasonic pulse and transmitted by the quartz transmitter, picked up by the receiver, then reshaped and amplified and returned to the transmitter. An alternative version converted the electrical impulses into stress waves, which travelled down a length of nickel wire. This version was called a magnetostrictive store, from the method of transferring the impulses to and from the nickel, using coils similar to electromagnets. The main problem with the delay line was that is was sequential storage and did not allow random access. Despite this it was popular with post-war British computer developers.

A second approach that did allow random access to the information stored was to make use of the ability of materials like phosphor to retain electrostatic charges 10. Information was stored as areas of charge, usually on a phosphor CRT. This form of storage suffered from the problem of the charges leaking. A number of different approaches were used in an attempt to reduce this leakage but the most successful was the Williams tube, invented in late 1946. This approach refreshed the charges periodically, before they could leak. A low powered electron reading beam was used to detect whether the charge stored represented a zero or a one. The power of the beam was then increased and the bit refreshed. The trick used by Williams was that he found that it was possible for the reading beam to detect what the next bit of information was slightly before the beam reached it 15. This method of storage was frequently used in early American computers.

The final method was to make use of the magnetic properties of materials. There were two types. The first was the magnetic drum. This is based on a very similar idea to the floppy discs of today. A rotating drum was covered in a magnetic material and used as a backing store, in conjunction with a faster form of storage technology to act as the main memory. The other type was the magnetic core memory. However, this technology was still being developed in America during the period covered by this essay.

In the remainder of this section, I will briefly look at a number of early British computers, created during the valve era of computing, covering roughly the time period from the end of World War II to the mid 1950's, when the transistor was beginning to come into common use.

5.2 THE NPL ACE AND PILOT ACE

After the war, Turing joined the NPL. While he was there, he began working on a design for a universal computer, that he called the Automatic Computing Engine, or ACE. By the end of 1946, Turing had already produced five different versions of a design 11. The design process eventually went on for three years, with the construction finally starting on a 'test assembly' in late 1947.

The design of the ACE was different to modern computer architectures, in that it had no accumulator 11. Instead, it consisted of about 200 mercury delay lines, each one capable of storing 32 32-bit long binary words. The system was designed so that commands moved data from a source to a destination, usually the delay lines. A number of 'special purpose' data manipulation delay lines were used for data manipulation operations, such as addition. Most of the special delay lines could only hold 32-bit word, but some capable of holding two words (actually 65-bits) 10 were also used.

Initially, the work on the construction of the ACE was subcontracted 15, but in 1948, the head of the NLP electronics section decided to leave. At about the same time, Turing, unhappy with the rate of progress being made on the ACE, also left and took a job at the University of Manchester. The replacement head of the electronics section, suggested that the electronics section took over the construction of the test assembly. Work on this was then abandoned and instead a simplified version of the ACE, called the Pilot ACE was constructed. Construction began in early 1949 6 and the machine was working by the end of May 1950 11 (although it was not actually completed until late 1951, mainly because of a number of modifications that were made 6). It consisted of a reduced amount of delay line memory, initially allowing 128 32-bit words, although this was later increased to 352 words and a 4096 word drum store was also added 10. It also had the special function delay lines, allowing addition and subtraction, and also had a special unit to allow multiplication to be performed. The Pilot ACE used fewer valves than Colossus, approximately 800, compared to the 1500 used in the earlier computer. The internal clock frequency was 1 megacycle, making it the fastest British computer of this period.

The Pilot ACE was programmed mainly using punched cards 10. It was also possible to enter programs using an array of 32 switches, and in fact the first program to be run on the Pilot ACE was entered in this way, due to the fact that the punched card system had not yet been set up. Output was also from punched cards, and an array of lights.

The Pilot ACE was put into full-time service in early 1952, following some  modifications, including the increases in memory size already mentioned. English Electric eventually re-engineered the Pilot ACE, making a number of changes to the arrangement of the temporary stores 6, with an aim to make it more reliable and easier to maintain. This caused the number of valves used to increase to 1450 10. The machine was marketed by English Electric as the DEUCE in 1955, and further developed to give first the DEUCE 2 in late 1955, then the DEUCE 2A in 1957. By 1964, 31 DEUCE machines had been sold.

Development continued on the full version of the ACE, and it was eventually finished by 1959 11. It contained 7000 valves and had a 48-bit word size. By this time, technology had moved on, from mercury delay lines to core memory, and the technology of the ACE was rapidly becoming outdated. However, as has been mentioned, the Pilot ACE and ACE computers were the fastest of the time - any two 9 digit decimal numbers could be multiplied together in under 1/5000th of a second 10.

The original Pilot ACE was closed down in May 1956 6. The front panel and some of the electronics can now be found in the Science Museum in London 15.

5.3 EDSAC AND LEO

The EDSAC (Electronic Discrete Variable Automatic Calculator) 10 was developed at Cambridge University between about 1947 and mid-1949, the first automatic calculation being performed in May 1949 11. The design was inspired in part by the American EDVAC system. Some of the development was funded by J. Lyons and Co. Ltd 15, who were hoping that an accounting problem they were suffering could be solved by a computer.

EDSAC used mercury delay lines as its main memory 10. In fact, one of the team building the computer had worked on delay lines during World War II, on application involving the clarity of radar displays. The lines, 32 in all, were used to store 35-bit words 15. In addition, a number of additional tubes acted as registers in the central processor unit. The input and output were via paper tape systems. The computer consisted of approximately 3000 valves. The clock speed was half that of the Pilot ACE. Each 35-bit word could be treated either as a 'long' 35-bit word or as two 'short' 17-bit words, with a separating bit between them (this bit was lost, due to electronic switching times) 4. A 17-bit word consisted of 16-bits of data, and 1 sign bit.

In structure, EDSAC was more like the systems of today that the ACE, in that it contained an accumulator for use in arithmetic operations. It was easier to program than the ACE computers, because a symbolic assembly language was provided, which allowed users to write meaningful instructions using alphabetic characters. These were then translated to machine instructions internally by EDSAC 10. Altogether there were 18 such instructions, including add, subtract, left and right accumulator shifting and conditional branches, but no unconditional branch. EDSAC also allowed programs to consist of relocatable subroutines, that could be linked together at load time. A graduate student called David Wheeler developed a system where the current location in a program could be stored, then the system jumped to one of the subroutines, executed it and when it was finished, jumped back to the point at which it had left the program 15. EDSAC was finally closed down in July 1956, following about 6 years of regular use 11.

Like the Pilot ACE, EDSAC was used commercially, albeit in a re-engineered form, as the LEO (Lyons Electronic Office). This is acknowledged to be the first commercial use of a computer for data processing 15. Construction of the first LEO started in mid 1949 7 and was finished in 1951. This machine was in used for nearly 14 years, and a number other systems based on it were developed. Eventually, in 1954 a new company, LEO Computers, was formed to build these. The LEO was similar to EDSAC, but used a number of different possible input and output devices, and a number of special instructions were included to allow the selection of different devices.

5.4 THE MANCHESTER COMPUTERS AND FERRANTI

The Manchester Automatic Digital Machine (MADM), or Mark I as it came to be called, was developed at Manchester University, starting in late 1946 10. It started as a project to develop electrostatic charge storage devices using standard CRTs, the form of storage that later became known as a Williams tube after one member of the development team, F.C. Williams (the other being Tom Kilburn).

By late 1947, the Williams tube was at a sufficient stage of development to be tested, and a small prototype computer was built, using a Williams tube capable of storing 32 32-bit numbers 10. This computer took input from a bank of switches and output to a monitoring display screen. Like EDSAC it contained an accumulator, although this was based on a CRT rather than a mercury line 15. It first ran in mid June 1948, executing a factoring program.

Following the success of this first program, a large number of improvements and expansions were made, including in April 1949 the increase of the word length to 40-bits, and the addition of a drum store, to supplement the Williams tube, which was in turn improved so that it was able to 128 words 5. This two level memory structure was one of two innovations that was taken up by other computer manufacturers, the other being the inclusion of index registers 10.

As of April 1949, the Mark I contained roughly 1300 valves and had a 26 operation instruction set. The method of programming the Mark I was partly developed by Alan Turing 10 and was based on teleprinter code. Consequently, many of the programmers found this system quite difficult to use. The system was used to solve a number of 'realistic' problems, including solving problems connected with guided weapons.

The original system was shut down in August 1950, to make way for further computer developments. However, the designs had been given almost two years previously to Ferranti Ltd, in Manchester. This was mainly due to a visit by Sir Ben Lockspeiser, then the government Chief Scientist, to see the Mark I in action, in October 1948 10. He was so impressed that he initiated a government contract in November 5 with Ferranti to create production versions of the Mark I. A total of nine Ferranti Mark I and Mark I Star computers (the Mark I Star being an additional development of the Mark I, funded by the National Research Development Corporation) starting in 1951 and finishing in 1957. In fact, the first Ferranti Mark I delivered in February 1951 was installed in Manchester University.

As the Ferranti Mark I was overpowered for the university's needs, outside computing projects were encouraged. These included the development (originally as a hobby) of what is probably the world's first compiler, by Dr Alec Glennie, in 1952, called 'autocode' 10, and the Mark I Autocode language, an attempt to make programming the Mark I 'user-friendly'.

One of the Mark I Star computers was delivered in April 1955 to the Atomic Weapons Research Establishment (AWRE) 13 at Aldermaston, Berkshire (now the Atomic Weapons Establishment), after a number of scientists had been sent to both Cambridge and Manchester to take part in early development work. It was joined by a DEUCE computer in 1956 16. In fact, Dr Glennie eventually retired from AWRE a couple of years ago.

Further computers were developed in Manchester, starting in 1951 with the Mark II, or MEG (Megacycle Engine), which was finished in 1954 and was the prototype of a later Ferranti computer.

6. CONCLUSIONS

The computer as it exists today owes at least part of its existence to Alan Turing and Colossus. Turing's theories made Colossus possible, while Colossus itself proved that it was possible to build a general purpose digital machine, even though it was originally designed for a special purpose. Without these, all three computers would probably still have been built, but the NPL Pilot and Manchester Mark I, would almost certainly have existed in a different form, due to the contributions made to them by ex-Colossus personnel, notably Turing, Newman and Flowers.

REFERENCES

1. Gareth Ashurst, Pioneers of Computing, Frederick Muller Limited, 1983.

2. Stan Augarten, Bit by Bit, An Illustrated History of Computers, George Allen and Ullwin Limited, 1985.

3. B.V. Bowden (ed), Faster Than Thought: A Symposium on Digital Computing Machines, Sir Issac Pitman & Sons Limited, 1964.

4. M. Campbell-Kelly, Programming the EDSAC: Early Programming Activity at the University of Cambridge, Annals of the History of Computing vol. 2, no. 1, 1980, pp. 7-37.

5. M. Campbell-Kelly, Programming the Mark I: Early Programming Activity at the University of Manchester, Annals of the History of Computing vol. 2, no. 2, 1980, pp. 130-168.

6. M. Campbell-Kelly, Programming the Pilot ACE: Early Programming Activity at the National Physical Laboratory, Annals of the History of Computing vol. 3, no. 2, 1981, pp. 133-162.

7. M. Campbell-Kelly, The Development of Computer Programming in Britain (1945 to 1955), Annals of the History of Computing vol. 4, no. 2, 1982, pp. 121-139.

8. Frank Carter, Codebreaking with the Colossus Computer, The Bletchley Park Trust Reports, November 1996.

9. F.H. Hinsley and A. Stripp (ed), Code Breakers: The Inside Story of Bletchley Park, Oxford University Press, 1994.

10. S. Lavington, Early British Computers, Manchester University Press, 1980.

11. Ed. N. Metropolis, J. Howlett and Gian-Carlo Rota, A History of Computing in the

Twentieth Century, Academic Press, 1980.

12. R. Moreau, The Computer Comes of Age, MIT Press, 1984.

13. C. Russ, History of Computing in Building E2 (Issue 1, Draft A), AWE SCD Software Advisory Service, 1995.

14. Nigel West, GCHQ: The Secret Wireless War 1900-86, Weidenfeld and Nicolson, 1986.

15. Michael R. Williams, A History of Computing Technology, Prentice-Hall, 1985.

16. Unknown author, Digital Computing Facilities as AWRE Aldermaston, AWRE Reproduction Unit, unknown publication date.